HERIOT-WATT UNIVERSITY

DEPARTMENT OF COMPUTING AND ELECTRICAL ENGINEERING

22.3MB1 Electromagnetics

Tutorial 1

1.	Determine the electric field at a distance r from a uniform, straight, infinite line charge of density λ Cm−1.

[Er=λ2πεr]

2.	Determine the electric field in the vicinity of a uniform infinite plane sheet of charge of density σ Cm−2.

[En=σ2ε]

3.	Using Gauss's Law determine an expression for the electric field intensity inside an infinitely long cylindrical charge distribution of radius a and uniform charge density ρCm−3 immersed in a medium of permittivity e.

[Er=ρr2ε]

- Consider a parallel plate capacitor with a sheet of mica of the same area, A,

between its plate. The plate separation is d and the thickness of the mica is a (d>a).

Ignore fringing to derive that the capacitance is:

C=ε0Ad+a(1−εmεm)

where m is the relative dielectric constant of the mica.

Now derive the capacitance for i) a = 0 ii) a = d

What is the interest of using materials such as Mica to realise the capacitor instead of air?

- Consider a infinite conductor. Calculates its inductance per unit length.

Hint: Fist calculate the Field, then the flux linkage (careful here) and finally deduce L.

[L=μ08π]

6	A toroid has a mean diameter DAv and an effective area Aeff. If the toroidal core has a relative permeability r, then find the expression for the inductance if a conductor is wrapped around the core N times. A current given by i=20sin(50t) (A) is passed throught the coil. Find the voltage induced across the coil.

[L=410−7μrN2AeffDAv]	    [V=410−5μrN2AeffDAvcos(50t)]