HERIOT-WATT UNIVERSITY

DEPARTMENT COMPUTING AND ELECTRICAL ENGINEERING

Electromagnetics

Tutorial 3

1.	Starting with Maxwell's equations in differential form derive the wave equation

∇2H=μoε∂2H∂t2

2.	Determine the relative permittivity of a medium having an intrinsic impedance of 60 pW assuming that the medium is non-magnetic.  What will be the velocity of a TEM wave in such a medium?  If the instantaneous maximum power transmitted by the wave is 25 watts through an area 10 metres square, determine the magnitude of E in .

[εr=4, u=1.5×108ms−1, 21.7Vm−1]

3.	A plane e-m wave travels in a lossless medium with a velocity u=1μoε.  By considering the rate of flow of energy per unit area show that

S=E×H

is a valid measure of the instantaneous power flow.

4.

A coaxial cable has a steady current I flowing in it and the conductors are assumed to have zero resistance.  By taking the following steps

(i)	Ampere's Law to find magnetic field

(ii)	Gauss's Law to find E

(iii)	V=E⋅dr to find V

(iv)	application of Poynting's theorem

(v)	integration of the Poynting vector over the coaxial line cross-section

show that the total power flow is W = VI.  Consider the implication that the power flow is through the dielectric space and not through the conductors.

5.	A sinusoidal plane electromagnetic wave is transmitted through a medium whose breakdown strength is 30 k and whose relative permittivity is 4.  Determine the maximum possible r.m.s. power flow density, and the peak value of the associated magnetising force.

[, 159 ]

6.	Consider a cylindrical resistor of radius a and length L carrying a d.c. current I due to the applied voltage V.  By considering the magnetic and electric fields associated with I and V show that the evaluation of the Poynting vector produces the well known result that the power dissipated P = VI.