HERIOT-WATT UNIVERSITY

DEPARTMENT OF COMPUTING AND ELECTRICAL ENGINEERING

22.3MB1 Electromagnetics

Tutorial 5

1.	Briefly discuss the effect of conductivity on a plane TEM wave propagating in a lossy medium, and sketch the variation of sinusoidally varying E and H field components for such a wave travelling in the z-direction.  [from Question B2, 1993].

Define the term “skin depth”, d

Given that the propagation constant gfor a plane TEM wave in a lossy medium is

g=jwms+jwe=α+jβ

derive, for the special case of a “good conductor”, expressions for the following:

(i)	attenuation constant:		α

(ii)	phase constant:		β

(iii)	skin depth:			d

(iv)	intrinsic impedance:		h

The E-field strength of a uniform plane TEM wave in free space is 1 Vm−1and the frequency is 10 kHz.  This downward travelling wave is incident on a large flat area of ground with s=10−2Sm−1 and er=5.  Check that the “good conductor” approximation is valid and calculate the E-field, (a) at the surface, and (b) at a depth of 100m.

2.	Given the wave equation for an electromagnetic wave propagating in a source-free region, as:

Ñ2E−−em¶2E−¶t2−sm¶E−¶t=0

show that, for propagation along the z-axis only, and for sinusoidally varying fields, the equation for E− may be written as:  [from Question B1, 1991].

¶2E−¶z2−g2E−=0

where

g2=jwms+jwe

The flat side of a metallic object is covered with a 10 mm thick uniform layer of non-conducting material to reduce the amplitude of 3 GHz radar reflections.

If the material has mr=er=5−j5 determine the amplitude of the reflected E-field component relative to the incident value.

3.	With reference to suitable diagrams show the instantaneous time and space relationship between electric field and magnetic field for a sinusoidal plane TEM wave propagating in an unbounded region of free space.  [from Question B1, 1989].

Indicate the significance of the Poynting Vector for the wave, both graphically and in vector notation.

Briefly, how is the E and H pattern modified for a wave propagating in a conducting medium?

By considering a laser beam as a plane TEM wave, estimate the diameter of the beam of a 1 kW laser that would cause voltage breakdown in air.  Assume that air has a dielectric breakdown strength of 3 MVm−1.

[Note:	ho (free space) = 120pΩ]