# Question April 2000

(a)

Maxwell's two 'curl' equations are as follows:

∇×E=−∂Bdt

∇×H=J+∂Ddt

Make use of these equations to derive the following wave equation for the electric field component of a time-changing electromagnetic wave propagating in a uniform, linear, isotropic, non-magnetic region of conductivity  Sm-l, permittivity,  Fm-l, and permeability Hm-1-1:

∇2E=μoσ∂E∂t+μoε∂2E∂t2

If the time variation of E is sinusoidal show that the above equation may be expressed as:

∇2E−γ2E=0

where

γ2=jωμσ+jωε

If the time variation of E is restricted to variation along the z-axis only, show that the above wave equation may be further simplified to

Show that E=Eoe−γzis a valid solution to this simplified wave equation.

Writing  as γ=α+jβobtain expressions for  and  for the extreme cases of

- a ‘perfect dielectric’ and

- a ‘good conductor’

Consider the following four uniform regions with regard to a 1MHz plane TEM wave. decide for which case(s) a ‘good conductor’ or a ‘perfect dielectric’ approximation may apply, and for such case(s) calculate the following parameters:

(a)	attenuation constant  (dB m-1)

(b)	phase constant  (degree m-1)